MN Plotting Program

MNPLOT will plot the far-field response calculated by MN directly on

your graphics screen.  Both azimuth and elevation patterns are plotted, using

either polar or rectangular coordinates.  MNPLOT will work with a Color

Graphics Adapter, Enhanced Graphics Adapter, or Hercules Graphics Card.  You

can execute MNPLOT by itself to view plots made previously, or you can use the

P command while running MN.  This automatically generates a plot file for the

antenna under analysis, calls MNPLOT to display the plot, and then returns to

MN when you are done.

If you are using an EGA or HGC, a second plot file may be selected for

pattern comparison.  The new plot is drawn into the second graphics buffer, and

the screens may be switched back and forth instantly with a single key stroke,

without having to wait for the plots to be redrawn.  This permits a precise

instantaneous pattern comparison, revealing effects of small changes in an

antenna design that would not be noticed in a side by side comparison of

printed plots.  Either of the antenna plots may be replaced by a third, and so

on, allowing multiway comparisons two at a time.  This feature is not available

for the CGA because it lacks a second graphics buffer.

Running MNPLOT

To run MNPLOT do the following, according to your display type:

HGC:	The MNHGC graphics driver must be loaded before plotting.  Simply

execute MNHGC.  This needs to be done only once per boot, so MNHGC

may be put in your AUTOEXEC.BAT file for convenience.

CGA:	Load GRAFTABL and GRAPHICS once per boot (or put them in your

AUTOEXEC.BAT file).  These are standard DOS programs.  They will

improve the CGA graphics characters and allow you to transfer the

CGA plots to your printer.

EGA:	The EGA requires no preliminaries.

When executed, MNPLOT automatically determines which type of graphics

display card is present, and then configures itself accordingly.  The MNPLOT

features vary according to the display type.

You may give the name of the plot file on the command line when

invoking MNPLOT, otherwise you will be prompted for it.  You need not type the

.PLT extension, it will be provided automatically.

The following keys are active in various modes during the plot:

<Enter>	Switch between azimuth and elevation plots

P		Switch between polar and rectangular coordinates

L		Switch the polar plot scale between log dB and linear dB

Y		Change the rectangular plot Y-axis gain cutoff

N		Normalize the plot of the lower-gain antenna to the gain scale

of the higher-gain antenna; pressing again reverts to normal

A		Select another antenna and turn off plot compare mode

C		Select another antenna and use plot compare mode (not for CGA)

<Space> 	Switch between two comparison plots (not for CGA)

1-6		Change display colors (EGA only)

S		Save display colors (EGA only)

<PrtSc>	Print screen (Press <Shift><PrtSc> or <Shift><F10>)

<Esc>		Exit the plot program

F1		Activate the help panel, an abbreviated	version of this list

The help panel only lists keys which are active at the time the F1 key

is pressed.  For example, the Y key is only listed when in rectangular mode.

Similarly the N, S, C, <Space>, and 1-6 keys are never listed for the CGA,

since the CGA hardware cannot handle these functions.

The A command will turn off compare mode, enabling you to examine the

patterns of single antennas more rapidly.  Also note that the C command can be

used repeatedly to compare multiple antennas two at a time.

Whenever you are prompted for a plot filename you may press <Enter> to

obtain a listing of the available plot files.

Polar Plots

The azimuth polar plot is displayed with the MN +X direction toward the

top of the screen, and the +Y direction to the left.  Zero azimuth angle is at

the top of the screen, 90 deg to the left, 180 at the bottom, and 270 to the

right.  Elevation plots have the +Z direction (overhead) at the top of the

screen, and the +X axis to the right.  Unidirectional antennas, if aimed in the

+X direction, will have their major lobe pointing upward on the azimuth plot,

and toward the right on the elevation plot.  This arrangement allows you to

distinguish azimuth plots from elevation plots at a glance.

You may select either of two radial scales for a polar plot. The

standard ARRL log dB scale causes lower level sidelobes to be compressed toward

the center of the pattern, emphasizing the major lobe shape.  Almost all

antenna patterns in amateur radio publications use this scale, so it provides a

way to compare the antennas you develop using MN to existing designs.  It also

produces patterns having familiar shapes.  The center of the plot is minus

infinity dB (no signal) with this scale, but there is little plot area below

-40 dB.  The other polar scale is a linear dB scale, with -50 dB at the center

of the plot, and much more area for signals between -20 and -50 dB than the log

dB scale.  This scale is useful for examining small sidelobes which are hard to

see in a log dB plot.

The dots forming the radial lines (the ones which are not multiples of

30 deg) are spaced every 2 dB.  The dots forming the circles are spaced at

multiples of 1 deg.  These calibrations allow you to read directivity values

off the antenna pattern with good accuracy.  Using them you may never need to

use the MN D command to generate tabulated pattern data in a .RUN file.

If the antenna is in free space MNPLOT will draw a 360 deg elevation

polar plot.  If it is over ground only the upper 180 deg hemisphere is shown.

To save time, MN normally generates data only for 0 to 180 deg in

azimuth, and 0 to 180 deg (the overhead half plane) in elevation.  In this case

MNPLOT assumes symmetry around the X axis, and draws the right half of the

azimuth plot from the data for the left half.  Similarly, for free space

antennas the lower half of the elevation plot simply duplicates the pattern of

the upper half.  However, for asymmetrical antennas you can force MN to

generate data for the entire 360 deg plane by using "360" as a parameter when

entering the G command to MN.  Both azimuth and elevation data will be

generated for 360 deg (elevation data will cover 180 deg if not free space).

The polar plots will be perfectly circular on a monitor adjusted for

normal aspect ratio.  If they look elliptical you should adjust your vertical

height control.  This control is located at the rear of most monitors.

Rectangular Plots

Rectangular or X-Y plots may be selected with the P command.  This

coordinate system reveals small sidelobe detail even better than the linear dB

polar plot, but the overall shape of the pattern is lost.  The X-axis is

azimuth or elevation angle from 0 to 180 or 360 degrees, and the Y-axis is the

antenna response in dB using a linear scale.  The Y-axis cutoff, or lower limit

in dB, may be changed using the Y command.  Because this parameter is always a

negative number, you may enter it without the minus sign for convenience.  The

Y-axis cutoff is constrained between -1 and -100 dB.  The Y-axis is easiest to

interpret when the cutoff value is a multiple of 10 deg, but you may enter any

value and the scale and grid will be drawn correctly.  The horizontal grid

lines have dots every 2 or 4 deg.  The vertical grid lines have dots every .02,

.1, .25, .5, 1, or 2 dB depending on the Y-axis cutoff value.  The X-axis is

180 deg wide unless the plot file contains independent data for the whole 360

deg plane.

Plotting within MN

If MNPLOT is present in the current directory, MN will generate a plot

file whenever you enter the P command.  The root name of the plot file will be

the same as that of the current antenna file, with the extension .PLT.  When

experimenting with an antenna you may wish to save more than one plot file,

either for later examination or for immediate comparison using the plot compare

feature.  To generate a plot file with a new name simply append the name as a

parameter to the P command.  The .PLT extension need not be typed as MN will

add it automatically.

If you are only interested in viewing the azimuth response you need not

wait for the elevation data generation to complete after entering the P

command.  Simply press <Esc> to abort the elevation search, and the azimuth

plot will appear.  The plot file will only contain azimuth data, but will

otherwise plot normally.  You may later enter P, wait for the elevation search

to complete, and MN will add the elevation data to the existing plot file.

Adding the parameter "360" to the G command will cause the azimuth and

elevation data to be generated for the entire 360 deg plane.  This is useful

for plotting the response of asymmetrical antennas, such as wire antennas

erected to take advantage of randomly positioned tall trees.  A subsequent P

command, after a "G 360", will generate the plot file, and call MNPLOT.

Some highly-directive antennas will plot better if data is generated

every 1 deg instead of the usual 2 deg.  To cause this use the parameter "1"

with the G command.  This parameter may be used at the same time as the "360"

parameter.  Parameter order and spacing are not important as MN just looks for

a "1" and a "360" somewhere on the G command line.  These parameters are not

available with the P command to avoid confusion with the P parameter that

renames plot files.  To plot with the "360" and "1" parameters, do G first with

the parameters, then P.

Plot data is generated with a dynamic range of 100 dB and with a

resolution of .01 dB.  The elevation angle is annotated on azimuth plots only

if it is nonzero, and the same holds for the azimuth angle on elevation plots.

DOS Environment Variables for MNPLOT

You may tell MNPLOT to display gains in dBi rather than dBd by using

the following DOS command:

SET DB=dBi

Any combination of upper and lower case letters can be used.

To specify a directory to be used as a plot library do the following:

SET PLTLIB=Directory

MNPLOT will list the plot files in this directory and those in the

current directory when started without a filename on the command line.  You may

load a plot file from the plot library by simply entering the filename, the

directory need not be given.  This also applies if you specify the plot

filename as a command line parameter when invoking MNPLOT.  For more

information on SET commands see MN.DOC.

EGA Colors

Keys 1 through 6 will change the screen colors while you are viewing

the EGA display.  After selecting the colors you prefer, you may save the

changes by pressing the S key.  The color codes will be saved in the file

MNCOLORS.EGA.  When you next run MNPLOT the saved color set will be read back

in.  If MNCOLORS.EGA is not found MNPLOT will use bright white on a black

background for everything.

The color keys are:

1	Background color

2	Antenna pattern color

3	Grid color

4	Grid label color

5	Annotation color

6	Help panel color

Making Hardcopies of Plots

You may press <Shift><PrtSc> to print the current screen.  On some

keyboards MNPLOT may not recognize the key marked PrtSc.  If this is the case

you may use <Shift><F10> instead.  It is normal for the polar plots to come out

somewhat elliptical on paper.  For the EGA and HGC, if you start the first plot

at the top of the page, a second plot may be put on the same page without any

manual realignment of the printhead.  Simply call up the second plot and press

<Shift><PrtSc> again.  This is a nice way to save both the azimuth and

elevation plots for a single antenna, or to compare two antennas.  MNPLOT does

not respond to <Shift><PrtSc> or <Shift><F10> with the CGA, or perform any

printing function.  Instead, DOS will intercept <Shift><PrtSc> and print your

plot if GRAPHICS was loaded before starting MNPLOT.