(Comp.sources.hp48)

Item: 52 by perre@picea

Author: [Per Stenius]

Subj: Circut Simulator v2.61

Date: Sun Feb 16 1992

Csim 2.61 SHORT DESCRIPTION AND MANUAL 12/17/91 (c) Per Stenius

RELEASE NOTE

This version differs from the previous (2.3, 2.5, 2.51, 2.6) in the

following:

Version 2.3:

- Lossless transmission line for AC analysis at a single freq. point. (T)

- The functions CV(node) and CI(branch) that fetch the voltage/current of

given node/branch in the previous solution point. CV = ControlVoltage,

can be used in other sources to define any functional dependency of a

voltage/current in the circuit. Note that there is a delay of 'tstep'

when using these functions.

- The program 'iterdc' for iterative dc solving to be used with nonlinear

components.

- Additional examples of circuits in manual.

- New sample circuit.

Version 2.5, 2.51, (and previous versions):

- ABCD-matrix for two-port added.

- Minor changes in code for improved speed (during setup)

- BOTH NODES GND error removed, BOTH NODES SAME used instead.

Version 2.6 (and previous):

- ABCD, y and z matrix two-ports can be used in DC analysis if all matrix

elements are real (i.e. the imaginary part must be zero)

- A->L converts array into list (for easier handling of matrices)

INTRODUCTION

This text describes a simple circuit simulator called Csim for the HP48.

It makes DC, AC and transient simulations and supplies the user with all

matrices used. The method used is modified nodal analysis, and thus

elements such as an inductor or ideal voltage source require that a current

branch is also specified together with the nodes. As soon as a setup is

done, single analyses can be made with the 'dc', 'ac' or 'tran' sub-

programs. All subprograms return the result as a vector, whereas Csim

provides the user with a plot in transient and AC analysis. Note that for

Csim to work correctly, the 'node' variable should be defined (either a

node or branch number) when plotting a result. Note also that the RES

variable (in PLOTR) should be 0. Finally, if SYM (in MODES) is not set

Csim does not work correctly.

For a demo on how Csim works do the following:

- download the code (creates the directory CSIM in the current directory)

- enter the directory CSIM and enter the custom menu (the button marked CST

between the PRG and VAR buttons on your HP48)

- press [Csim]:

for: Setup? Y  press: <enter>

for: Analysis? (D,A,T) press: T <enter>

for: Sweep range?

:tstart:0

:tstep:0

:tstop:1          press: <backspc> 5 <enter> i.e. :tstop:5

after which a time-domain plot is drawn (if you have something that

you previously have plotted, press [CLRSC] before doing this).

Press <ON> to exit the graph environment.

Now, press 3 <left-shft> [node] <enter> and redo the above ([node] is

a variable in the custom menu).

Finally, press the CST button, <next> [CIR] [CIR->] to see the circuit

description.

Here is an explanation on the custom menu of Csim (the custom menu is

obtained by pressing the button marked CST on your HP48, and contains

those variables and programs you need for using Csim):

Csim - the simulator program. Runs setup (if requested) and a single

analysis. For ac, a single run directly from the custom menu

provides a fast solution in one freq-point. When choosing T

for transient analysis either tstep (time resolution) or tstop

should be given. If both are given tstep is used and tstop

ignored.

View  - the StackView application, for easy check on components. Use

ATTN (ON) to exit. You can also use the Interactive Stack the

HP48 provides (see manual p.70).

node  - the node or branch the value of which is wanted as a solution.

Used to GET the right value from the solution vector.

ymin, ymax

- define the picture y-axis (ac or tran analysis)

CLRSC - Clears the screen (simply the ERASE command)

outp  - a program that takes a vector from the stack and returns one value.

it can be used to plot a result that is a function of the values

in the solution vector. To be edited by the user.

(default << node GET >> )

CIR-> - takes a list of lists, such as the one used to store circuit

descriptions (see also CIR, ->CIR) and puts the lists in it to

the stack (inverse to ->CIR). Usage: Press a variable containing

a circuit description (e.g. CIR) and press CIR->.

->CIR - takes the circuit description used by Csim and puts it to a list,

that can be stored in a variable.

CIR   - when 'Setup' is run, the stack containing the circuit is stored in

this variable as a list. To use again, recall CIR and run 'CIR->'

(see above). Contains a sample circuit as default. Any circuit

description can be stored in a variable as a list of components

(which also are lists).

dc    - single DC analysis.  Requires that setup is done (the matrices are

ready). Takes no argument from stack and returns a solution vector.

ac    - single AC analysis. Requires that setup is done (the matrices are

ready) and that 'w' is specified (rads/s angular freq). Takes no

argument from stack and returns a solution vector. Note: Complex

values! When A is chosen in Csim, the actual program stored by STEQ

is 'acplot', which executes 'ac' and then 'outp'. The program

'outp' should take a vector from stack and return a single number.

E.g. << node GET ABS >> would return the absolute value of a node

voltage (or branch current) that is to be plotted.

A->L  - converts an array into a list.

tran  - single transient analysis. Takes one time step and returns a

solution vector. The method used is trapezoidal rule. When T is

chosen in Csim, the actual program stored by STEQ is 'tranBE' (or

'tranTR'). These return the result vector and call 'outp'. The

program 'outp' should take a vector from stack and return a single

number. E.g. << node GET >> would return the value of a node voltage

(or branch current) that is to be be plotted.

Setup - setup routine for the simulator. Creates and loads the matrices

needed and stores the stack as a list into CIR. Takes the circuit

description from the stack as an argument (only component declara-

tions are allowed on stack). Setup must be done once before analy-

zing, however, after that the matrices are ready to be used multi-

ple times. This should be remembered e.g. when calculating a DC

solution and thereafter starting a transient analysis from the

obtained results. In this case running Setup a second time would

zero the result vector. Setup also clears flag -3 (i.e. enables

SYM), sets flag -17 and clears flag -18 (i.e. sets radians mode).

w     - angular frequency (2*pi*f) rad/s.

G     - conductance matrix. Contains all real valued entries, i.e. those

caused by elements the values of which do not have an s or jw

factor.

C     - s-matrix. Contains all elements that have an s or jw factor.

Cc    - constant valued complex matrix. Can be used in AC analysis only,

at a single value of 'w' (angular frequency). Contains entries

from Z, Y, z, y (See Section on syntax).

W     - numerical values of the sources as a vector. This vector is

updated in every analysis point.

Wlist - the functions representing each source as a list, from which

the numerical values for 'W' are obtained.

Euler - specifies the method used in the tran analysis. If Euler = 1 then

backward Euler ('tranBE') is used (faster but more inaccurate), if

Euler = 0 then the trapezoidal rule is used, which is rather

accurate but slower ('tranTR' and 'tran').

iterdc

- if CV() or CI() are used in DC analysis, iterative solution is

required in order to obtain the correct solution. 'iterdc' can

be used for this after setup has been done. Note that very few

nonlinear circuits can actually be solved by iteration only.

Usually some linearization method must also be used, e.g. the

Newton-Raphson algorithm. Giving good initial guesses for node

voltages and branch currents in the X vector also helps.

THE SYNTAX USED TO DESCRIBE A CIRCUIT

The syntax by which the components are entered is (NOTE! Each circuit

needs a ground node and its number is always 0 (zero)):

{R node1 node2 numval branch} - resistance [ohm]

{G node1 node2 numval}        - conductance [mho]

{C node1 node2 numval}        - capacitance [F]

{L node1 node2 numval branch} - inductance [H]

{Y node1 node2 complexnumval} - admittance with a constant complex value

(re,im). In DC analysis each entry must

be real (im = 0).

{Z node1 node2 complexnumval} - impedance with a constant complex value

(re,im) In DC analysis each entry must

be real (im = 0).

{J node1 node2 funcval}       - indep. current source

{E node1 node2 funcval branch}

- indep. voltage source

{S node1 node2 branch}        - short circuit (the current is fetched by

branch GET). Can be used to define a current

branch for dependent sources.

{O in+ in- out+ out- outbranch}

- ideal opamp (out- should be ground,

outbranch returns the output current)

{M l1node1 l1node2 l2node1 l2node2 l1val l2val mval l1branch l2branch}

- transformer i.e. two inductors (l1, l2) with

mutual inductance (mval). The values required

are the four nodes, the value of l1, l2, mval

[H] and the branches of l1 and l2. The dots

for m are at l1node1 and l2node1.

{m l1branch l2branch mval}    - mutual inductance of mval [H]. As M, but can

be used to define e.g. three inductances

that all have mutual inductances. To do

this, define the 3 L:s and then 3 m:s be-

tween them. Note that m takes the branches

of the L:s. Make sure you specify the in-

ductors the right way (the branch of L runs

from n1 to n2). Note also that m is not a

component, it merely states a dependency

between two L:s that should be defined se-

parately. No checking is done that l1branch

and l2branch actually belong to L:s.

{T node1 node2 node3 node4 llval Zoval}

- lossless transmission line (to be used in AC

analysis only) of length ll (in wavelengths)

and with the characteristic impedance Zo.

Note that nodes 2 and 4 must have the same

value (equivalent pi-circuit used).

{g node1 node2 node3 node4 numval}

- voltage-controlled current source i.e.

transconductance. The source current is

from node3 to node4 and the controlling

voltage from node1 to node2.

{r node1 node2 node3 node4 numval branch1 branch2}

- current-controlled voltage source. Defines

a short circuit between node1 and node2

and a controlled voltage source between

node3 and node4 (node3 being the positive

node). The controlling current runs through

branch1 and the current of the source is

fetched from branch2. Branch1 must not be

a previously defined branch.

{p node3 node4 numval branch1 branch2}

- same as r but does not define a short

circuit between node1 and node2. Instead

branch1 must be a predefined branch (e.g.

that of a resistor or inductor).

{a node1 node2 node3 node4 numval branch}

- current-controlled current source. Defines

a short circuit between node1 and node2

and a controlled current source the current

of which runs from node3 to node4. The

controlling current runs through branch

which must not be previously defined.

{b node3 node4 numval branch} - same as a but does not define a short

circuit between node1 and node2. Instead

branch must be a predefined branch (e.g.

that of a resistor or inductor). Compare

with p.

{u node1 node2 node3 node4 numval branch}

- voltage-controlled voltage source. The

current through the source is fetched

from branch.

{y node1p1 node2p1 node1p2 node2p2 y11 y12 y21 y22}

- a two-port with y-parameters that are

constant complex values (re,im). In DC

analysis each entry must be real (im =

0).

{z node1p1 node2p1 node1p2 node2p2 z11 z12 z21 z22}

- a two-port with z-parameters that are

constant complex values (re,im). In DC

analysis each entry must be real (im =

0).

{A node1p1 node2p1 node1p2 node2p2 A B C D}

- a two-port with ABCD-parameters that

are constant complex values (re,im).

In DC analysis each entry must be real

(im = 0).

FUNCTIONS

CV(node)                      - returns previously calculated value of

the voltage of node. In code << X node

GET >>

CI(branch)                    - returns previously calculated value of

the current of branch. In code << X

branch GET >>, the same as CV().

In the above, nodes and branches are integer numbers. The ground node is

represented by 0. All nodes and branches should have a unique number and

they should be given in order e.g. nodes 0,1,2,3 and branches 4,5,6. These

numbers refer DIRECTLY to the position in the matrices/vectors. Thus the

4'th element in the result vector would be the current through branch 4 and

the first element is the voltage of node 1. To help remembering the syntax,

you could e.g. have a variable y with the following contents:

y

{y n1p1 n2p1 n1p2 n2p2 y11 y12 y21 y22}

NOTE: The two-port y, z and ABCD parameters are valid in DC analysis ONLY

IF ALL THE ENTRIES in the corresponding matrix are real.

The following components form equivalent circuits:

{S 1 2 5}

{p 3 4 100 5 6} is the same as

{r 1 2 3 4 100 5 6},

{S 1 2 5}

{b 3 4 100 5} is the same as

{a 1 2 3 4 100 5},

{L 1 2 0.1 5}

{L 3 4 0.2 6}

{m 5 6 0.05} is the same as

{M 1 2 3 4 0.1 0.2 0.05 5 6}.

Finally, an example of usage for transient analysis:

(This is how your stack should look)

{E 1 0 '10*SIN(10*t)' 4}

{C 1 2 0.01}

{L 2 3 1 5}

{R 3 0 10 6}

This defines a RLC-circuit with nodes 1,2,3 (and ground) and current

branches 4,5,6. The current through branch 4 is equal to the current

through the ideal voltage source E, the current through branch 5 equals

the current through the inductor L, and the current through branch 6

equals the current through the resistor R. The values of the components

(which ALWAYS must be numerical) are 10 ohms, 1 henry and 0.01 farads.

The voltage source has a time-dependent value (used in transient analysis).

If 'node' is set to 3 the voltage over the resistor R is plotted in

transient analysis. On the other hand, 'outp' could be written as << DUP

1 GET SWAP 2 GET - >> to return the voltage between nodes 1 and 2 as a

result. Note that sources (E,J) may have functional values (should be

suitable for the analysis requested! Time dependency for transient

analysis and 'w' dependency (angular freq) for AC). When running Csim,

the stack may ONLY contain component declarations!

Press the CST button and then [View]. Now press ATTN (low-left corner, i.e.

ON). Press [Csim], press <enter> on "Setup? Y", press D (or T), <enter> on

analysis.

An example for AC analysis (using A in Csim):

{E 2 0 1 3}

{G 2 1 1}

{C 1 0 1}

Set 'node' equal to 1 and write 'outp' equal to << node GET ABS >>. Se-

lect A on analysis and choose wstart 0 and wstop 10 (ymin = 0, ymax = 1).

Another example for AC analysis (using 'w' = 1 and 'ac'):

{J 0 1 10}

{G 3 0 1}

{G 2 0 1}

{Z 1 0 (0,-1)}

{M 3 1 2 1 2 1 0.5 4 5}

Setup

1 left-shft w ac

The currents through the transformer are the 4'th and 5'th elements in the

result vector. Here's an example involving a transistor for which we have

the y-parameters y11 = 0.001, y12 = -j0.0001, y21 = 0.1 and y22 = 0.0001.

Its base is at node 1, emitter at gnd (node 0) and collector at node 2.

{J 0 1 1}

{Z 1 0 1E3}       @ resistance of 1 kohms

{Z 2 0 1E3}

{Z 1 2 (0,-1000)} @ capacitance of -j1000 ohms

{y 1 0 2 0 1E-3 (0,-1E-4) 0.1 1E-4}

Setup

ac 2 GET ABS (returns |Uo/Jin| = 884.035 V/A)

For the use of the lossless transmission line we have the following ex-

ample (analysis can be made at a single frequency point only, at which ll

is valid):

{J 0 1 1}

{Z 2 0 (75,-69)}

{T 1 0 2 0 0.583 50}

Setup ac 1 GET

returns (25.2092, -34.5800) which is the input impedance (J = 1) of a loss-

less transmission line of the length 0.583 wavelengths (at some frequency)

and with the characteristic impedance of 50 ohms, terminated with a load of

(75,-69) ohms.

DC analysis:

{J 0 1 1}

{R 1 0 1E3 3}

{p 2 0 10 3 4}

The voltage of node 2 (the 2'nd element in the result vector) should be

10V. The current through the controlled voltage source should be 0A (the

4'th element in the result vector).

For iterative DC solving of  circuits, consider these two examples:

{E 1 0 1 3}

{G 1 0 1}

{G 1 2 1}

{J 0 2 'SQ(CV(1))'}

and

{J 0 1 1}

{G 1 0 1}

{G 1 2 1}

{E 2 0 'SQ(CV(1))' 3}

Setup iterdc

The first circuit converges after only two iterations, but the second one

requires several hundred to reach the exact solution ([[1] [1] [0]] in the

second case). Good initial guesses may help, and also the use of e.g. the

Newton-Raphson algorithm (see Vlach-Singhal; Computer Methods for Circuit

Analysis and Design). Note that 'iterdc' should actually also be used at

every time step in transient analysis to avoid the delay when using CV()

and CI().

A simple small signal model for a transistor (B-1 C-2 E-3):

{R 1 3 1E3 4}

{G 2 3 0.0001}

{b 2 3 100 4}

A voltage source used as an digital inverter; if the voltage of node 2 is

higher than 2.5 volts, then the voltage of node 3 is 0 volts, otherwise

5 volts (note that there is a delay of one 'tstep').

{E 3 0 'IFTE(CV(2)>2.5,0,5)' 4}

Including nonlinear components as such would make solving the matrix equ-

ation system a tedious process. It would also make the analysis MUCH slower.

However, it could be done.

ERROR MESSAGES

These are the only error messages in this program. Note that there are not

many error checking routines provided, so the user should be careful when

entering the circuit description. For any strange behaviour or false re-

sults, please email me directly and explain what occurred.

SYNTAX ERROR - an error occurred in 'Setup' while Csim was loading the

matrices. Check the circuit description and the component

that is first on stack. See also section on syntax.

NEGATIVE NODE NO. - a negative number was given as a node number. Check the

the first component on stack.

BOTH NODES SAME - a component was specified having two nodes that were the

same value. To override this, use a short-circuit (S) between

these nodes.

ZERO VALUE OR BRANCH - a component with a value of zero was given or its

branch number was zero (which is reserved for the ground

node).

n2 MUST EQUAL n4 IN T - you have entered a transmission line in the circuit

with nodes 2 and 4 not equal. This is not allowed since the

equivalent pi-circuit to the transmission line is actually

used.

Im{Cc} \=/ 0 IN DC - you have specified components with complex values in

DC analysis.

A good way to avoid errors is to proceed systematically, e.g. in the

following way:

1) Choose one reference node to be ground (GND) and set its node number

to 0.

2) Assign the rest of the nodes a number each, in numerical order:

1,2,3,...

3) Find all the components that require a branch and assign each required

branch a number starting from the highest node number plus one.

!) The node and the branch numbers should follow eachother in numerical

order, with no 'gaps' in between (e.g. 0,1,2 are nodes and 3,4 are

branches).

4) Enter the circuit description component by component. Note that the

stack should only contain component descriptions! Check your stack

with 'View'.

5) Decide what results you need and edit 'outp' if necessary. Also, set

'node' to the correct value.

6) Run 'Setup' once and start analyzing! Remember to run 'Setup' whenever

you change your circuit or want to start from zero. Sometimes all you

need to do is to edit your X vector.

FREQUENTLY ASKED QUESTIONS

Some people have had trouble with the branch currents (the direction...)

so here's more on that:

In all components (J,E,L etc.) the current is defined to be FROM the first

node TO the second node. Thus no matter which way you put your source (E)

The value of the branch current remains the same (i.e. in the direction of

the U of the source) with respective to the source. In the sample RLC

circuit, when the first node of E and L is the same, the currents should

be the opposite. When the second node of E is the first node of L, the

currents are the same.

For E the situation looks like this:

--- U=E -->

node1 +  o->-( E )---o - node2

I

for L:

--- U1,2 ->

node1    o->- Ind ---o   node2

I

for J:

node1    o->-( J )---o   node2

I=J

for S:

--- U=0 -->

node1    o----->-----o   node2

I

This could be defined the other way around too, but in this case it isn't.

For two-ports, the node numbers are defined as

_________

node1    o->--|         |--<-o node3

|         |

|         |

|         |

|         |

node2    o----|_________|----o node4

Note that the direction of the currents is always towards the two-port.

This should be remembered when defining e.g. ideal opamps, controlled

sources and two-ports with y- or z-parameter representation.

>I can't get the transient analysis to work properly unless I do

>an entire setup first. If I do a transient plot, and then repeat it, I

>get different results, unless I run setup.

>

What happens is that unless you run setup, the transient analysis continues

from where it stopped (however, this time from the beginning of the screen).

Thus the new beginning should match with the previous end. As you might

have noticed, the transient analysis should always start from time=0. This

is due to the fact that solving this problem is an iterative process.

>It would be nice to be able to specify initial values for capacitors and

>inductors.

>

This can be done. The X vector contains the starting values, and is zeroed

at setup. However, nothing stops you from running a dc-analysis and then

running a tran analysis without a setup in between, thereby giving the re-

sults from the dc-analysis as beginning values for the tran analysis. The

X vector can also be manually edited. To do this, run setup, then press

enter for analysis?, which stops the program. Edit the X, and run tran

analysis without setup. The execution of setup can be avoided by answering

something else than 'Y' at 'Setup?'.

>I'm interested in any references you used, for algorithms for circuit

>solving, this is something I've never really looked into before.

>

A good place to start is to look at a book called

Computer Methods for Circuit Analysis and Design

by Jiri Vlach and Kilshore Singhal (Van Nostrand Reinhold Company 1983,

ISBN 0-442-28108-0). Check out chapter 4 and 9. For transient analysis,

(which is normally done by Laplace tranforms when working manually) the

trapezoidal rule is pretty powerful. Also, for all the theory needed for

Csim, I have written a report called

A Tutorial on Developing a Simple Circuit Simulating Program

which I can email (ps-file format) to anybody interested upon request. It

is about 30 pages + 30 pages including this manual and the commented source

code for Csim.

FINAL REMARK

This simulator is not necessarily completely bug-free, so please report

to me for any strange behaviour. Note also that the transient analysis

methods are not necessarily stable for all values of time steps. Try an-

other time range or time step if this happens. If you get the system error

INV Error:

Infinite Result

in any analysis mode, this usually indicates that the component matrix

cannot be inverted. This error can sometimes be avoided by assigning the

nodes the values 0...n and the branches the values n+1...m. If this does

not help, please send me the circuit description you used.

I am happy to provide any further information on this program. Please

send also some comments on its appearance, suggestions on improvement

etc. Note that very little syntax checking is done (e.g. no node or

branch number checks!). I hope this short manual is sufficient, if not

please ask me directly via email.

copyright Per Stenius, Helsinki University of Technology.

email perre@aplac.hut.fi or pstenius@otax.tky.hut.fi

END_DOC