This is a driver for using the Tangent Technologies MacBridge board as an

externally clocked syncronous serial line (such as 64kb).  You must first do

some modifications to the board.  First, remove the faceplate and move the

connecter from J2 to J3 at the bottom of the board.  Next, remove the crystal

Y1.  Lastly, cut the jumper from JY1 to JY2 and solder a new jumper from JY2

to JY3.  Now, the pinout on the connector J3 is:

1 TxD +

2 TxD -

3 RxD +

4 RxD -

5 RxClk +

6 RxClk -

7 GND

8 TxClk +

9 TxClk -

Some interesting things about the board:

Rx DMA channel #1 (No Tx DMA channel unfortunately!)

Interrupt channel #3

DTRA enables tx driver

DTRB enables DMA

RTSA enables interrupts

RTSB enables I/O ready - wait line

Chn A CTL address = 0x340

Chn B CTL address = 0x341

Chn A data address = 0x342

Chn B data address = 0x343

Clear DMA request after last  = 0x344

half duplex DMA

xtal 3.6864 MHz