Both of these programs do time-varying

spectral warping. In each case, a warp

function is provided, consisting of N/2

values where is value is a multiplier of

the frequency in the corresponding FFT bin.

Tofu requires a second function which

specifies an instantaneous offset of the

warp function (ranging from 0-1).

Squeegie does not offset the warp function

but requires a second function specifying

the instantaneous  intensity of the warp

where 0.0 is no warping, 1. is full warping

and >1.0 is extra warping.

tofu:  dynamic spectral warping

tofu   [flags] < floatsams > floatsams

N:      fft length [1024]

R:      sampling rate [44100]

M:      window size in samples [4096]

D:      decimation factor in samples [512]

I:      interpolation factor in samples [512]

w:      warp function [wfunc]

o:      offset function [ofunc]

a:      permit aliasing

d:      duration of output [undefined]

g:      threshold generator [.001]

s:      synthesize analysis input

squeegie:  dynamic spectral warping

squeegie   [flags] < floatsams > floatsams

N:      fft length [1024]

R:      sampling rate [44100]

M:      window size in samples [4096]

D:      decimation factor in samples [512]

I:      interpolation factor in samples [512]

w:      warp function [wfunc]

o:      scaling function [ofunc]

a:      permit aliasing

d:      duration of output [undefined]

g:      threshold generator [.001]

s:      synthesize analysis input

Here's a tofu example:

(Now that the CARL software is on the streets, I don't feel

awkward about using their "gen" programs.)

medication> gen4 -L1024 0 1 -4  2000 .5 0 22050 .5 > wfunc

medication> gen1 -L667  0 0 1 1 > ofunc

medication> fromsf -H karlheinz_dig | tofu -N2048 -g.005 | tosf -R44100 hemiola_lips

you could even try the same data on squeegie:

medication> fromsf -H De_Sade.snd | squeegie | tosf -R44100 Skinner.snd

Hopefully you learned as much about naming soundfiles

as about tofu and squeegie.

Hey, thank you.