ntplot

BASIC INFORMATION

-----------------

Network Time Protocol (xntp) statistics graphing program

Requires: 	ntpq:  xntp query program

X11 window manager: X-Windows support software

room and access for a temporary file

Files:		ntplot.c: sole source.

ntplot.tmp: temporary file's name.

Written in C with calls to xlib, for UNIX environment.

By Charlie Ledogar, CERN CN/CS, July 1991.

Questions/comments welcomed; please mail to ledogar@eniac.seas.upenn.edu.

PURPOSE

-------

To make ntpq's statistics more clear, to keep a history of them for

long-term performance observations, to make problems wth the timeserver

immediately apparent and obvious.

Also: used as an exercise in C, X and UNIX programming by the author.

This program is completely dependent upon the xntpd and ntpq network

time software, and should not be considered out of that context.

COMPILING

---------

Link with xlib, m, and possible dnet.

following works at least in SUN:

cc -O -o ntplot ntplot.c -lm -lX11

If you are in SYSV machine add -DSYSV

cc -O -DSYSV -o ntplot ntplot.c -lm -lX11

STARTUP

-------

Syntax:

ntplot [-u UPDATE_PERIOD] [-t TICKTYPE] [-D] [-g WIDTHxHEIGHT]

[-p PATH_OF_NTPQ_EXECUTABLE] [-h HOSTNAME]

UPDATE_PERIOD : minimum delay between

updates, in seconds.          Default: 30s.

TICKTYPE : interval between timestamps;

one of 'd','h','m','s'.            Default: "h".

D: enables debugging output.

WIDTH, HEIGHT: dimensions of windows, in

pixels.  Should be at

least 150x150.                 Default: 320x200.

PATH_OF_NTPQ_EXECUTABLE: directory in which

a readable, executable

copy of ntpq resides.   Default: /usr/local/etc

HOSTNAME: name of ntp server to query.        Default: localhost

Required in environment:

DISPLAY x11 display client address:display number

-------------------------- --------------

DISPLAY

-------

After +-20 seconds of information polling, one window for each of xntp's

remote peers will appear.  Each window is divided vertically into 3

sections.  The top section shows a graph of the remote peer's reported

time offsets; the middle section shows a graph of the peer's time

dispersions; the bottom section shows a "confidence" bar graph, a

timestamp graph, and the peer's last non-clock reference.

The offset graph ranges from -300 to 300 milliseconds, a range into which

most offsets seem to fall.  A "good" offset is a low, steady one.

The dispersion graph ranges logarithmically from 1 to 100000 milliseconds.

"Good" dispersions are low.  If xntp loses contact momentarily with a

remote peer, the dispersion will jump to 64000, its maximum, then sink

down to normal values.  Frequent occurence of this could indicate a

problem: see xntp documentation.

The confidence graph is a reflection of the dispersion and offset; it is

a measure of how much consideration the xntpd daemon gave the remote peer

in it's clock selection algorithm.  The confidence can take one of 4 values:

0 = no consideration.

1 = determined to be an inaccurate time source (a "falseticker").

2 = a good time source.  The dispersion must be below 64 ms for this

"truechimer" status.

3 = the current xntpd synchronization peer (sys_peer); the best of the

"truechimers".

The confidence graph displays these values as line thicknesses

(1, 3, 5, or 7 pixels thick).

The timestamp graph gives a good idea of the timescale of the graphs.

Depending on the length of history the operator wishes to observe

(selected using the -d option), he/she may wish to label the timescale

with ticks every second, minute, hour, or day of the month.  This is

specified with the -t option: "-t s", "-t m", etcetera.

The "last non-clock reference" field is useful for identifying new or

preferred primary servers for reconfiguration.  For instance,

iis.ethz.ch was thought to be the only public primary NTP server in

Switzerland, but investigation of this field revealed that iis often

gave preference to the clock of dns.unige.ch (the latter was therefore

added as a peer to CERN's configuration).

SHUTDOWN

--------

Double-clicking the mouse in any window will terminate the program.

FEATURES OF THE PROGRAM

-----------------------

ie. things that were tricky to code:

Dynamic peer display:

ntplot responds immediately to any peer addition

or unconfiguration with addition or removal of the appropriate

window.  (THE MAXIMUM NUMBER OF PEERS DISPLAYABLE IS 16 but any more

than this is unwise anyway.)

Timestamps:

The timestamps are derived from the (hopefully accurate) system

clock with a closest-rollover algorithm.  If the program is

suspended, the timestamps will remain accurate upon continuation.

There is code for preventing timestamps from overlaying each other

in case of high delay-to-tick-frequency values.

Full-time event support:

Expose events result in window redrawing.  Supposedly, multiple expose

events of the same window are truncated to 1 event.  Double-clicking

the mouse in a window stops the program.

Low-overhead background query-program polling:

ntpq is run as a seperate process using "fork" to enable

the expose-event support.  If you are worried about processor load,

use a high update delay, or change the code to make the ntpq

calls nicer.

Minimal delay assurance:

The program will make sure that ntpq's data is processed before

calling ntpq again.  The actual ntpq delay is about 3 seconds

per peer plus system overhead (about 17 seconds total for 5

peers on a VAXstation 2000).  Note that this delay can drop to

milliseconds per peer if ntpq is called with the -n option.

I didn't bother making this a ntplot option, though, since

there's no real hurry, and the canonical names look nicer anyway.

DISCLAIMER

----------

The program is totally unstructured, and makes use of several system

calls that the programmer barely understands.  The code was unthoroughly

debugged and is barely portable.

COPYRIGHTS

----------

See source.

THANKS TO:

----------

Jean-Michel Jouanigot, John Gamble, Brian Carpenter, Per Hagen:

CERN Computing and Networking Division / Communications Systems.

Martyn Johnson: Univ. of Cambridge Computer Lab.

The creators and programmers of NTP and XNTP.

BUGS:

_____

This morning, I found that ntplot had hung after running for

3-4 days.  Not good!  I thought the problem might have been

because the maximum array size "wmax" had somehow been reached,

but it doesn't seem so.  However, I decided to release this

program anyway.  Anyone with information, please mail to

ledogar@eniac.seas.upenn.edu.

FUTURE SUGGESTIONS:

-------------------

Neat graphs such as those in RFC1129 (would have been easy, but

fairly useless for our server).

Alternating inversion of colors for screen-saver purposes?

A parent window with other information.

A version of X that creates smaller executables!