FFTPACK

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

version 4  april 1985

a package of fortran subprograms for the fast fourier

transform of periodic and other symmetric sequences

by

paul n swarztrauber

national center for atmospheric research  boulder,colorado 80307

which is sponsored by the national science foundation

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

this package consists of programs which perform fast fourier

transforms for both complex and real periodic sequences and

certain other symmetric sequences that are listed below.

1.   rffti     initialize  rfftf and rfftb

2.   rfftf     forward transform of a real periodic sequence

3.   rfftb     backward transform of a real coefficient array

4.   ezffti    initialize ezfftf and ezfftb

5.   ezfftf    a simplified real periodic forward transform

6.   ezfftb    a simplified real periodic backward transform

7.   sinti     initialize sint

8.   sint      sine transform of a real odd sequence

9.   costi     initialize cost

10.  cost      cosine transform of a real even sequence

11.  sinqi     initialize sinqf and sinqb

12.  sinqf     forward sine transform with odd wave numbers

13.  sinqb     unnormalized inverse of sinqf

14.  cosqi     initialize cosqf and cosqb

15.  cosqf     forward cosine transform with odd wave numbers

16.  cosqb     unnormalized inverse of cosqf

17.  cffti     initialize cfftf and cfftb

18.  cfftf     forward transform of a complex periodic sequence

19.  cfftb     unnormalized inverse of cfftf

******************************************************************

subroutine rffti(n,wsave)

****************************************************************

subroutine rffti initializes the array wsave which is used in

both rfftf and rfftb. the prime factorization of n together with

a tabulation of the trigonometric functions are computed and

stored in wsave.

input parameter

n       the length of the sequence to be transformed.

output parameter

wsave   a work array which must be dimensioned at least 2*n+15.

the same work array can be used for both rfftf and rfftb

as long as n remains unchanged. different wsave arrays

are required for different values of n. the contents of

wsave must not be changed between calls of rfftf or rfftb.

******************************************************************

subroutine rfftf(n,r,wsave)

******************************************************************

subroutine rfftf computes the fourier coefficients of a real

perodic sequence (fourier analysis). the transform is defined

below at output parameter r.

input parameters

n       the length of the array r to be transformed.  the method

is most efficient when n is a product of small primes.

n may change so long as different work arrays are provided

r       a real array of length n which contains the sequence

to be transformed

wsave   a work array which must be dimensioned at least 2*n+15.

in the program that calls rfftf. the wsave array must be

initialized by calling subroutine rffti(n,wsave) and a

different wsave array must be used for each different

value of n. this initialization does not have to be

repeated so long as n remains unchanged thus subsequent

transforms can be obtained faster than the first.

the same wsave array can be used by rfftf and rfftb.

output parameters

r       r(1) = the sum from i=1 to i=n of r(i)

if n is even set l =n/2   , if n is odd set l = (n+1)/2

then for k = 2,...,l

r(2*k-2) = the sum from i = 1 to i = n of

r(i)*cos((k-1)*(i-1)*2*pi/n)

r(2*k-1) = the sum from i = 1 to i = n of

-r(i)*sin((k-1)*(i-1)*2*pi/n)

if n is even

r(n) = the sum from i = 1 to i = n of

(-1)**(i-1)*r(i)

*****  note

this transform is unnormalized since a call of rfftf

followed by a call of rfftb will multiply the input

sequence by n.

wsave   contains results which must not be destroyed between

calls of rfftf or rfftb.

******************************************************************

subroutine rfftb(n,r,wsave)

******************************************************************

subroutine rfftb computes the real perodic sequence from its

fourier coefficients (fourier synthesis). the transform is defined

below at output parameter r.

input parameters

n       the length of the array r to be transformed.  the method

is most efficient when n is a product of small primes.

n may change so long as different work arrays are provided

r       a real array of length n which contains the sequence

to be transformed

wsave   a work array which must be dimensioned at least 2*n+15.

in the program that calls rfftb. the wsave array must be

initialized by calling subroutine rffti(n,wsave) and a

different wsave array must be used for each different

value of n. this initialization does not have to be

repeated so long as n remains unchanged thus subsequent

transforms can be obtained faster than the first.

the same wsave array can be used by rfftf and rfftb.

output parameters

r       for n even and for i = 1,...,n

r(i) = r(1)+(-1)**(i-1)*r(n)

plus the sum from k=2 to k=n/2 of

2.*r(2*k-2)*cos((k-1)*(i-1)*2*pi/n)

-2.*r(2*k-1)*sin((k-1)*(i-1)*2*pi/n)

for n odd and for i = 1,...,n

r(i) = r(1) plus the sum from k=2 to k=(n+1)/2 of

2.*r(2*k-2)*cos((k-1)*(i-1)*2*pi/n)

-2.*r(2*k-1)*sin((k-1)*(i-1)*2*pi/n)

*****  note

this transform is unnormalized since a call of rfftf

followed by a call of rfftb will multiply the input

sequence by n.

wsave   contains results which must not be destroyed between

calls of rfftb or rfftf.

******************************************************************

subroutine ezffti(n,wsave)

******************************************************************

subroutine ezffti initializes the array wsave which is used in

both ezfftf and ezfftb. the prime factorization of n together with

a tabulation of the trigonometric functions are computed and

stored in wsave.

input parameter

n       the length of the sequence to be transformed.

output parameter

wsave   a work array which must be dimensioned at least 3*n+15.

the same work array can be used for both ezfftf and ezfftb

as long as n remains unchanged. different wsave arrays

are required for different values of n.

******************************************************************

subroutine ezfftf(n,r,azero,a,b,wsave)

******************************************************************

subroutine ezfftf computes the fourier coefficients of a real

perodic sequence (fourier analysis). the transform is defined

below at output parameters azero,a and b. ezfftf is a simplified

but slower version of rfftf.

input parameters

n       the length of the array r to be transformed.  the method

is must efficient when n is the product of small primes.

r       a real array of length n which contains the sequence

to be transformed. r is not destroyed.

wsave   a work array which must be dimensioned at least 3*n+15.

in the program that calls ezfftf. the wsave array must be

initialized by calling subroutine ezffti(n,wsave) and a

different wsave array must be used for each different

value of n. this initialization does not have to be

repeated so long as n remains unchanged thus subsequent

transforms can be obtained faster than the first.

the same wsave array can be used by ezfftf and ezfftb.

output parameters

azero   the sum from i=1 to i=n of r(i)/n

a,b     for n even b(n/2)=0. and a(n/2) is the sum from i=1 to

i=n of (-1)**(i-1)*r(i)/n

for n even define kmax=n/2-1

for n odd  define kmax=(n-1)/2

then for  k=1,...,kmax

a(k) equals the sum from i=1 to i=n of

2./n*r(i)*cos(k*(i-1)*2*pi/n)

b(k) equals the sum from i=1 to i=n of

2./n*r(i)*sin(k*(i-1)*2*pi/n)

******************************************************************

subroutine ezfftb(n,r,azero,a,b,wsave)

******************************************************************

subroutine ezfftb computes a real perodic sequence from its

fourier coefficients (fourier synthesis). the transform is

defined below at output parameter r. ezfftb is a simplified

but slower version of rfftb.

input parameters

n       the length of the output array r.  the method is most

efficient when n is the product of small primes.

azero   the constant fourier coefficient

a,b     arrays which contain the remaining fourier coefficients

these arrays are not destroyed.

the length of these arrays depends on whether n is even or

odd.

if n is even n/2    locations are required

if n is odd (n-1)/2 locations are required

wsave   a work array which must be dimensioned at least 3*n+15.

in the program that calls ezfftb. the wsave array must be

initialized by calling subroutine ezffti(n,wsave) and a

different wsave array must be used for each different

value of n. this initialization does not have to be

repeated so long as n remains unchanged thus subsequent

transforms can be obtained faster than the first.

the same wsave array can be used by ezfftf and ezfftb.

output parameters

r       if n is even define kmax=n/2

if n is odd  define kmax=(n-1)/2

then for i=1,...,n

r(i)=azero plus the sum from k=1 to k=kmax of

a(k)*cos(k*(i-1)*2*pi/n)+b(k)*sin(k*(i-1)*2*pi/n)

********************* complex notation **************************

for j=1,...,n

r(j) equals the sum from k=-kmax to k=kmax of

c(k)*exp(i*k*(j-1)*2*pi/n)

where

c(k) = .5*cmplx(a(k),-b(k))   for k=1,...,kmax

c(-k) = conjg(c(k))

c(0) = azero

and i=sqrt(-1)

*************** amplitude - phase notation ***********************

for i=1,...,n

r(i) equals azero plus the sum from k=1 to k=kmax of

alpha(k)*cos(k*(i-1)*2*pi/n+beta(k))

where

alpha(k) = sqrt(a(k)*a(k)+b(k)*b(k))

cos(beta(k))=a(k)/alpha(k)

sin(beta(k))=-b(k)/alpha(k)

******************************************************************

subroutine sinti(n,wsave)

******************************************************************

subroutine sinti initializes the array wsave which is used in

subroutine sint. the prime factorization of n together with

a tabulation of the trigonometric functions are computed and

stored in wsave.

input parameter

n       the length of the sequence to be transformed.  the method

is most efficient when n+1 is a product of small primes.

output parameter

wsave   a work array with at least int(2.5*n+15) locations.

different wsave arrays are required for different values

of n. the contents of wsave must not be changed between

calls of sint.

******************************************************************

subroutine sint(n,x,wsave)

******************************************************************

subroutine sint computes the discrete fourier sine transform

of an odd sequence x(i). the transform is defined below at

output parameter x.

sint is the unnormalized inverse of itself since a call of sint

followed by another call of sint will multiply the input sequence

x by 2*(n+1).

the array wsave which is used by subroutine sint must be

initialized by calling subroutine sinti(n,wsave).

input parameters

n       the length of the sequence to be transformed.  the method

is most efficient when n+1 is the product of small primes.

x       an array which contains the sequence to be transformed

wsave   a work array with dimension at least int(2.5*n+15)

in the program that calls sint. the wsave array must be

initialized by calling subroutine sinti(n,wsave) and a

different wsave array must be used for each different

value of n. this initialization does not have to be

repeated so long as n remains unchanged thus subsequent

transforms can be obtained faster than the first.

output parameters

x       for i=1,...,n

x(i)= the sum from k=1 to k=n

2*x(k)*sin(k*i*pi/(n+1))

a call of sint followed by another call of

sint will multiply the sequence x by 2*(n+1).

hence sint is the unnormalized inverse

of itself.

wsave   contains initialization calculations which must not be

destroyed between calls of sint.

******************************************************************

subroutine costi(n,wsave)

******************************************************************

subroutine costi initializes the array wsave which is used in

subroutine cost. the prime factorization of n together with

a tabulation of the trigonometric functions are computed and

stored in wsave.

input parameter

n       the length of the sequence to be transformed.  the method

is most efficient when n-1 is a product of small primes.

output parameter

wsave   a work array which must be dimensioned at least 3*n+15.

different wsave arrays are required for different values

of n. the contents of wsave must not be changed between

calls of cost.

******************************************************************

subroutine cost(n,x,wsave)

******************************************************************

subroutine cost computes the discrete fourier cosine transform

of an even sequence x(i). the transform is defined below at output

parameter x.

cost is the unnormalized inverse of itself since a call of cost

followed by another call of cost will multiply the input sequence

x by 2*(n-1). the transform is defined below at output parameter x

the array wsave which is used by subroutine cost must be

initialized by calling subroutine costi(n,wsave).

input parameters

n       the length of the sequence x. n must be greater than 1.

the method is most efficient when n-1 is a product of

small primes.

x       an array which contains the sequence to be transformed

wsave   a work array which must be dimensioned at least 3*n+15

in the program that calls cost. the wsave array must be

initialized by calling subroutine costi(n,wsave) and a

different wsave array must be used for each different

value of n. this initialization does not have to be

repeated so long as n remains unchanged thus subsequent

transforms can be obtained faster than the first.

output parameters

x       for i=1,...,n

x(i) = x(1)+(-1)**(i-1)*x(n)

+ the sum from k=2 to k=n-1

2*x(k)*cos((k-1)*(i-1)*pi/(n-1))

a call of cost followed by another call of

cost will multiply the sequence x by 2*(n-1)

hence cost is the unnormalized inverse

of itself.

wsave   contains initialization calculations which must not be

destroyed between calls of cost.

******************************************************************

subroutine sinqi(n,wsave)

******************************************************************

subroutine sinqi initializes the array wsave which is used in

both sinqf and sinqb. the prime factorization of n together with

a tabulation of the trigonometric functions are computed and

stored in wsave.

input parameter

n       the length of the sequence to be transformed. the method

is most efficient when n is a product of small primes.

output parameter

wsave   a work array which must be dimensioned at least 3*n+15.

the same work array can be used for both sinqf and sinqb

as long as n remains unchanged. different wsave arrays

are required for different values of n. the contents of

wsave must not be changed between calls of sinqf or sinqb.

******************************************************************

subroutine sinqf(n,x,wsave)

******************************************************************

subroutine sinqf computes the fast fourier transform of quarter

wave data. that is , sinqf computes the coefficients in a sine

series representation with only odd wave numbers. the transform

is defined below at output parameter x.

sinqb is the unnormalized inverse of sinqf since a call of sinqf

followed by a call of sinqb will multiply the input sequence x

by 4*n.

the array wsave which is used by subroutine sinqf must be

initialized by calling subroutine sinqi(n,wsave).

input parameters

n       the length of the array x to be transformed.  the method

is most efficient when n is a product of small primes.

x       an array which contains the sequence to be transformed

wsave   a work array which must be dimensioned at least 3*n+15.

in the program that calls sinqf. the wsave array must be

initialized by calling subroutine sinqi(n,wsave) and a

different wsave array must be used for each different

value of n. this initialization does not have to be

repeated so long as n remains unchanged thus subsequent

transforms can be obtained faster than the first.

output parameters

x       for i=1,...,n

x(i) = (-1)**(i-1)*x(n)

+ the sum from k=1 to k=n-1 of

2*x(k)*sin((2*i-1)*k*pi/(2*n))

a call of sinqf followed by a call of

sinqb will multiply the sequence x by 4*n.

therefore sinqb is the unnormalized inverse

of sinqf.

wsave   contains initialization calculations which must not

be destroyed between calls of sinqf or sinqb.

******************************************************************

subroutine sinqb(n,x,wsave)

******************************************************************

subroutine sinqb computes the fast fourier transform of quarter

wave data. that is , sinqb computes a sequence from its

representation in terms of a sine series with odd wave numbers.

the transform is defined below at output parameter x.

sinqf is the unnormalized inverse of sinqb since a call of sinqb

followed by a call of sinqf will multiply the input sequence x

by 4*n.

the array wsave which is used by subroutine sinqb must be

initialized by calling subroutine sinqi(n,wsave).

input parameters

n       the length of the array x to be transformed.  the method

is most efficient when n is a product of small primes.

x       an array which contains the sequence to be transformed

wsave   a work array which must be dimensioned at least 3*n+15.

in the program that calls sinqb. the wsave array must be

initialized by calling subroutine sinqi(n,wsave) and a

different wsave array must be used for each different

value of n. this initialization does not have to be

repeated so long as n remains unchanged thus subsequent

transforms can be obtained faster than the first.

output parameters

x       for i=1,...,n

x(i)= the sum from k=1 to k=n of

4*x(k)*sin((2k-1)*i*pi/(2*n))

a call of sinqb followed by a call of

sinqf will multiply the sequence x by 4*n.

therefore sinqf is the unnormalized inverse

of sinqb.

wsave   contains initialization calculations which must not

be destroyed between calls of sinqb or sinqf.

******************************************************************

subroutine cosqi(n,wsave)

******************************************************************

subroutine cosqi initializes the array wsave which is used in

both cosqf and cosqb. the prime factorization of n together with

a tabulation of the trigonometric functions are computed and

stored in wsave.

input parameter

n       the length of the array to be transformed.  the method

is most efficient when n is a product of small primes.

output parameter

wsave   a work array which must be dimensioned at least 3*n+15.

the same work array can be used for both cosqf and cosqb

as long as n remains unchanged. different wsave arrays

are required for different values of n. the contents of

wsave must not be changed between calls of cosqf or cosqb.

******************************************************************

subroutine cosqf(n,x,wsave)

******************************************************************

subroutine cosqf computes the fast fourier transform of quarter

wave data. that is , cosqf computes the coefficients in a cosine

series representation with only odd wave numbers. the transform

is defined below at output parameter x

cosqf is the unnormalized inverse of cosqb since a call of cosqf

followed by a call of cosqb will multiply the input sequence x

by 4*n.

the array wsave which is used by subroutine cosqf must be

initialized by calling subroutine cosqi(n,wsave).

input parameters

n       the length of the array x to be transformed.  the method

is most efficient when n is a product of small primes.

x       an array which contains the sequence to be transformed

wsave   a work array which must be dimensioned at least 3*n+15

in the program that calls cosqf. the wsave array must be

initialized by calling subroutine cosqi(n,wsave) and a

different wsave array must be used for each different

value of n. this initialization does not have to be

repeated so long as n remains unchanged thus subsequent

transforms can be obtained faster than the first.

output parameters

x       for i=1,...,n

x(i) = x(1) plus the sum from k=2 to k=n of

2*x(k)*cos((2*i-1)*(k-1)*pi/(2*n))

a call of cosqf followed by a call of

cosqb will multiply the sequence x by 4*n.

therefore cosqb is the unnormalized inverse

of cosqf.

wsave   contains initialization calculations which must not

be destroyed between calls of cosqf or cosqb.

******************************************************************

subroutine cosqb(n,x,wsave)

******************************************************************

subroutine cosqb computes the fast fourier transform of quarter

wave data. that is , cosqb computes a sequence from its

representation in terms of a cosine series with odd wave numbers.

the transform is defined below at output parameter x.

cosqb is the unnormalized inverse of cosqf since a call of cosqb

followed by a call of cosqf will multiply the input sequence x

by 4*n.

the array wsave which is used by subroutine cosqb must be

initialized by calling subroutine cosqi(n,wsave).

input parameters

n       the length of the array x to be transformed.  the method

is most efficient when n is a product of small primes.

x       an array which contains the sequence to be transformed

wsave   a work array that must be dimensioned at least 3*n+15

in the program that calls cosqb. the wsave array must be

initialized by calling subroutine cosqi(n,wsave) and a

different wsave array must be used for each different

value of n. this initialization does not have to be

repeated so long as n remains unchanged thus subsequent

transforms can be obtained faster than the first.

output parameters

x       for i=1,...,n

x(i)= the sum from k=1 to k=n of

4*x(k)*cos((2*k-1)*(i-1)*pi/(2*n))

a call of cosqb followed by a call of

cosqf will multiply the sequence x by 4*n.

therefore cosqf is the unnormalized inverse

of cosqb.

wsave   contains initialization calculations which must not

be destroyed between calls of cosqb or cosqf.

******************************************************************

subroutine cffti(n,wsave)

******************************************************************

subroutine cffti initializes the array wsave which is used in

both cfftf and cfftb. the prime factorization of n together with

a tabulation of the trigonometric functions are computed and

stored in wsave.

input parameter

n       the length of the sequence to be transformed

output parameter

wsave   a work array which must be dimensioned at least 4*n+15

the same work array can be used for both cfftf and cfftb

as long as n remains unchanged. different wsave arrays

are required for different values of n. the contents of

wsave must not be changed between calls of cfftf or cfftb.

******************************************************************

subroutine cfftf(n,c,wsave)

******************************************************************

subroutine cfftf computes the forward complex discrete fourier

transform (the fourier analysis). equivalently , cfftf computes

the fourier coefficients of a complex periodic sequence.

the transform is defined below at output parameter c.

the transform is not normalized. to obtain a normalized transform

the output must be divided by n. otherwise a call of cfftf

followed by a call of cfftb will multiply the sequence by n.

the array wsave which is used by subroutine cfftf must be

initialized by calling subroutine cffti(n,wsave).

input parameters

n      the length of the complex sequence c. the method is

more efficient when n is the product of small primes. n

c      a complex array of length n which contains the sequence

wsave   a real work array which must be dimensioned at least 4n+15

in the program that calls cfftf. the wsave array must be

initialized by calling subroutine cffti(n,wsave) and a

different wsave array must be used for each different

value of n. this initialization does not have to be

repeated so long as n remains unchanged thus subsequent

transforms can be obtained faster than the first.

the same wsave array can be used by cfftf and cfftb.

output parameters

c      for j=1,...,n

c(j)=the sum from k=1,...,n of

c(k)*exp(-i*(j-1)*(k-1)*2*pi/n)

where i=sqrt(-1)

wsave   contains initialization calculations which must not be

destroyed between calls of subroutine cfftf or cfftb

******************************************************************

subroutine cfftb(n,c,wsave)

******************************************************************

subroutine cfftb computes the backward complex discrete fourier

transform (the fourier synthesis). equivalently , cfftb computes

a complex periodic sequence from its fourier coefficients.

the transform is defined below at output parameter c.

a call of cfftf followed by a call of cfftb will multiply the

sequence by n.

the array wsave which is used by subroutine cfftb must be

initialized by calling subroutine cffti(n,wsave).

input parameters

n      the length of the complex sequence c. the method is

more efficient when n is the product of small primes.

c      a complex array of length n which contains the sequence

wsave   a real work array which must be dimensioned at least 4n+15

in the program that calls cfftb. the wsave array must be

initialized by calling subroutine cffti(n,wsave) and a

different wsave array must be used for each different

value of n. this initialization does not have to be

repeated so long as n remains unchanged thus subsequent

transforms can be obtained faster than the first.

the same wsave array can be used by cfftf and cfftb.

output parameters

c      for j=1,...,n

c(j)=the sum from k=1,...,n of

c(k)*exp(i*(j-1)*(k-1)*2*pi/n)

where i=sqrt(-1)

wsave   contains initialization calculations which must not be

destroyed between calls of subroutine cfftf or cfftb

["send index for vfftpk" describes a vectorized version of fftpack]